2023

Annual Drinking Water Quality Report

Brown’s Creek Water Company

DHEC #4420002

We're pleased to present to you this year's Annual Quality Water Report. This report is designed to inform you about the quality water and services we deliver to you every day. Our constant goal is to provide you with a safe and dependable supply of drinking water. We want you to understand the efforts we make to continually improve the water treatment process and protect our water resources. We are committed to ensuring the quality of your water. Our water is purchased treated surface water (Broad River) from the City of Union.	 If you have any questions about this report or concerning your water utility, please contact Will Teaster.  We want our valued customers to be informed about their water utility. If you want to learn more, please attend any of our regularly scheduled meetings. They are held on the second Thursday at 6:00pm at the Brown’s Creek Water Office, 4287 Lockhart Hwy, Union, SC.  A Source Water Assessment plan has been completed for our system by SCDHEC. If you do not have internet access, please contact Will Teaster at 864-424-9820 to make arrangements to review this document or to ask other questions.

Brown’s Creek routinely monitors for constituents in your drinking water according to Federal and State laws. This table shows the results of our monitoring for the period of January 1st to December 31st, 20223  As water travels over the land or underground, it can pick up substances or contaminants such as microbes, inorganic and organic chemicals, and radioactive substances.  All drinking water, including bottled drinking water, may be reasonably expected to contain at least small amounts of some constituents.  It's important to remember that the presence of these constituents does not necessarily pose a health risk.

In this table you will find many terms and abbreviations you might not be familiar with. To help you better understand these terms we've provided the following definitions:

Non-Detects (ND) - laboratory analysis indicates that the constituent is not present.

Parts per million (ppm) or Milligrams per liter (mg/l) - one part per million corresponds to one minute in two years or a single penny in $10,000.

Parts per billion (ppb) or Micrograms per liter - one part per billion corresponds to one minute in 2,000 years, or a single penny in $10,000,000.

Action Level - the concentration of a contaminant that, if exceeded, triggers treatment or other requirements that a water system must follow.

Treatment Technique (TT) - A treatment technique is a required process intended to reduce the level of a contaminant in drinking water.

Maximum Contaminant Level (MCL) - The “Maximum Allowed” (MCL) is the highest level of a contaminant that is allowed in drinking water.  MCLs are set as close to the MCLGs as feasible using the best available treatment technology.

Maximum Contaminant Level Goal (MCLG) -The “Goal”(MCLG) is the level of a contaminant in drinking water below which there is no known or expected risk to health.  MCLGs allow for a margin of safety.

Maximum Residual Disinfectant Level (MRDL) - The highest level of a disinfectant allowed in drinking water.  There is convincing evidence that addition of a disinfectant is necessary for control of microbial contaminants.

Maximum Residual Disinfectant Level Goal (MRDLG) – The level of a drinking water disinfectant below which there is no known or expected risk to health.  MRDLGs do not reflect the benefits of the use of disinfectants to control microbial contaminants.

| Regulated Substances Detected in Finished Drinking Water |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Substance | Violation Y/N |  | Level Detected | Unit Measurement | MCLG |  |  | MCL |  | Likely Source of Contamination |  |  |
| Chlorine (2023) | N |  | 1.3 Range 1.3-1.3 | ppm | 4 |  |  | 4 |  | Water additive used to control microbes |  |  |
| Inorganic Contaminants (City of Union 2023) |  |  |  |  |  |  |  |  |  |  |  |  |
| Fluoride (2021) | N |  | .40 Range 0.35-35 | ppm | 4 |  |  | 4 |  | Erosion of natural deposits; water additive which promotes strong teeth; discharge from fertilizer and aluminum factories |  |  |
| Nitrate (as Nitrogen) | N |  | 0.54 Range 0.54-0.54 | ppm | 10 |  |  | 10 |  | Runoff from fertilizer use; leaching from septic tanks, sewage; erosion of natural deposits |  |  |
| Sodium** Unregulated contaminent | N |  | 14.0 | ppm | N/A |  |  | N/A |  | Naturally occurring. |  |  |
| LEAD AND COPPER TEST RESULTS (Brown’s Creek 2021) |  |  |  |  |  |  |  |  |  |  |  |  |
| Contaminant | Violation Y/N | 90th percentile |  | Unit Measurement |  |  | Action Level |  | Sites over action level |  |  | Likely Source of Contamination |
| Copper (2021) | N | .19 |  | ppm |  |  | 1.3 |  | 0 |  |  | Corrosion of household plumbing systems; erosion of natural deposits; leaching from wood preservatives |
| Lead(2021) | N | 0E-9 |  | ppb |  |  | 15 |  | 0 |  |  | Corrosion of household plumbing systems; Erosion of natural deposits |
| Volatile Organic Contaminants (2023) |  |  |  |  |  |  |  |  |  |  |  |  |
| Haloacetic Acids (HAAs) | N | 24.0 Range 17.0-32.0 |  | ppb |  | 60 |  |  | N/A |  | By-product of drinking water disinfectant |  |
| TTHM (Total trihalomethanes) | N | 63 Range 21.7-114.5 |  | ppb |  | 80 |  |  | N/A |  | By-product of drinking water chlorination |  |

Violation Table

| Total Trihalomethanes (TTHM) |  |  |  |
| --- | --- | --- | --- |
| Some people who drink water containing trihalomethanes in excess of the MCL offer many years may experience problems with their liver, kidneys, or central nervous systems, and may have an increased risk of getting cancer. |  |  |  |
| Violation Type | Violation Begin | Violation End | Violation Explanation |
| MCL, LRAA | 10/01/2023 | 12/31/2023 | Water samples showed that the amount of this contaminant in our drinking water was above its standard (called a maximum contaminant level and abbreviated MCL) for the period indicated. |

| Revised Total Coliform Rule(RTCR) |  |  |  |
| --- | --- | --- | --- |
| The Revised Total Coliform Rule(RTCR) seeks to prevent waterborne diseases caused by E.coli, E.coli are bacteria ehose presence indicate the water may be contaminated with human or animal wastes. Human pathogens in these wastes can cause short term effect such diarrhea, cramps, nausea, head and other symptoms. They may pose a greater health risk for infants, young children and the elderly |  |  |  |
| Violation Type | Violation Begin | Violation End | Violation Explanation |
| MONITORING,ROUTINE,MINOR (RTCR) | 07/01/2023 | 07/31/2023 | We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our drinking water during the period indicated. |

| Surface Water Treatment Rule(SWTR) |  |  |  |
| --- | --- | --- | --- |
| The Surface Water Treatment Rule seeks to prevent waterborne diseases caused by viruses,Legionella, and Giardia lamblia. The rule requires that water systems filter and disinfect water from surface water sources to reduce the occurrence of unsafe levels of these microbes. |  |  |  |
| Violation Type | Violation Begin | Violation End | Violation Explanation |
| MONITORING ,RTN/RPT MAJOR(SWTR-FILTER) | 07/01/2023 | 07/31/2023 | We failed to test our drinking water for the contaminant and period indicated. Because of this failure, we cannot be sure of the quality of our drinking water during the period indicated. |

. We’re proud that your drinking water meets or exceeds all Federal and State requirements. We have learned through our monitoring and testing that some constituents have been detected. The EPA has determined that your water IS SAFE at these levels.

If present, elevated levels of lead can cause serious health problems, especially for pregnant women and young children.  Lead in drinking water is primarily form materials and components associated with service lines and home plumbing. Brown’s Creek Water Company is responsible for providing high quality drinking water but cannot control the variety of materials used in plumbing components.  When your water has been sitting for several hours, you can minimize the potential for lead exposure by flushing your tap for 30 seconds to 2 minutes before using water for drinking or cooking.  If you are concerned about lead in your drinking water, you may wish to have your water tested.  Information on lead in drinking water, testing methods, and steps you can take to minimize exposure is available from the Safe Drinking Water Hotline or at http://www.epa.gov/safewater/lead.

All sources of drinking water are subject to potential contamination by substances that are naturally occurring or man made. These substances can be microbes, inorganic or organic chemicals and radioactive substances. All drinking water, including bottled water, may reasonably be expected to contain at least small amounts of some contaminants.  The presence of contaminants does not necessarily indicate that the water poses a health risk. More information about contaminants and potential health effects can be obtained by calling the Environmental Protection Agency’s Safe Drinking Water Hotline at 1-800-426-4791.

Some people may be more vulnerable to contaminants in drinking water than the general population. Immuno-compromised persons such as persons with cancer undergoing chemotherapy, persons who have undergone organ transplants, people with HIV/AIDS or other immune system disorders, some elderly, and infants can be particularly at risk from infections. These people should seek advice about drinking water from their health care providers. EPA/CDC guidelines on appropriate means to lessen the risk of infection by cryptosporidium and other microbiological contaminants are available from the Safe Drinking Water Hotline (800-426-4791).